To run these example, MATLABR2022b, Deep Learning Toolbox, Mapping Toolbox, Matlab Report Generator, Statistic and Machine Learning Toolbox, and the EnergyPlus Version 8.3 are required.

- Example 1 USA

Zoning the states of Florida, Georgia, and Tennessee.

All settings are included in the file USA.zon.

- Files required:

|  | File | Location |
| --- | --- | --- |
| - Input file | USA.zon | simzoning\ |
| - Shape file of the States of Florida, Georgia, and Tennessee | USA_States.shp | simzoning\GISfiles\AreaOfStudy\ |
| - Weather files of the area of study and surroundings | 87 .EPW files | simzoning\Weatherfiles\ |
| - Idf. files | AptMidRise1A.idf AptMidRise2A.idf AptMidRise3A.idf AptMidRise4A.idf | simzoning\IDFs\USA_IDFS |
| - EnergyPlus Version | 8-3-0 |  |
| - Alternative method for comparison | DegreeDays_USA.SHP | simzoning\GISfiles\CZ_Methods_Comparison\ |

- Steps

- Create a folder C:/simzoning to unzip simzoning files.

- Confirm the path of EnergyPlus Version 8.3 installed in the computer. If necessary, rewrite the path in the USA.zon file used as input data to run this example.

- Call simzoning with the USA.zon file as input data from MATLAB.

| Case study summary |  |
| --- | --- |
|  | 87 Epws |
|  | 4 models 2 Performance indicators (Annual energy demand for cooling and heating.) 4 Zones Number of processors: 4 Time estimation 6 hours |

- Expected results

A Region with 4 Zones considering 2 Zoning resolution. A) Clustering based on points, b) clustering based on interpolated data.

Figure 1  Clustering considering points and interpolated data.

Figure 2 Cooling and heating variation per zone of a random model

Figure 3 Clustering based on a regular grid of interpolated data (Altitude, latitude, and longitude interpolation method)

Figure 4 Cooling and heating variation per zone of a random model

Figure 5 MPMA of clustering results compared to the degree days zoning for the region under analysis.

See PDF file Florida Georgia and Tennessee_AltLatLon_Report.pdf in the output folder.

simzoning/Outputs/Florida Georgia and Tennessee_CaseStudy/ for further details about expected results.